Tom Gradel - Homework #3

The following are images in case you don't want to run the code

Noise1 - Original;

Noise 1 - Smooth Filter

Noise 1 - Median Filter

Noise 1 - Dilation Filter

Noise 1- Erosion Filter

Noise 1 - Opening Filter

Noise 1 - Closing Filter

Noise 2 - Original

Noise 2 - Smooth Filter

Noise 2 - Median Filter

Noise 2 - Dilation Filter

Noise 2- Erosion Filter

Noise 2 - Opening Filter

Noise 2 - Closing Filter

Finally, here is the code:

% CIS601 - Tom Gradel - Homework 3 for image_no = 1:2 if (image_no == 1) filename = 'noise_1.gif'; else filename = 'noise_2.gif'; end I = imread(filename); % create smoothing filter f = ones(3) ./ 9; FI = filter2(f,double(I)); figure, imshow(I);title(['original ' filename]); figure, imshow(uint8(FI)); title(['smoothed ' filename]); % pad array [row,col]=size(I); ipad = [I(1,:); I; I(row,:)]; ipad = [ipad(:,1) ipad ipad(:,col)]; % create median filter im_median = zeros(row,col); im_dilation = zeros(row,col); im_erosion = zeros(row,col); for r = 1:row for c = 1:col m = double(ipad([r+1 r+2],[c+1 c+2])); im_median(r,c) = median(m(:)'); im_dilation(r,c) = max(m(:)'); % dilation filter = max im_erosion(r,c) = min(m(:)'); % erosion filter = min end end figure, imshow(uint8(im_median)); title(['median ' filename]); figure, imshow(uint8(im_dilation)); title(['dilation ' filename]); figure, imshow(uint8(im_erosion)); title(['erosion ' filename]); % create opening filter = dilation o erosion ipad = [im_erosion(1,:); im_erosion; im_erosion(row,:)]; ipad = [ipad(:,1) ipad ipad(:,col)]; im_opening = zeros(row,col); for r = 1:row for c = 1:col m = double(ipad([r+1 r+2],[c+1 c+2])); im_opening(r,c) = max(m(:)'); end end figure, imshow(uint8(im_opening)); title(['opening ' filename]); % create closing filter = erosion o dilation ipad = [im_dilation(1,:); im_dilation; im_dilation(row,:)]; ipad = [ipad(:,1) ipad ipad(:,col)]; im_closing = zeros(row,col); for r = 1:row for c = 1:col m = double(ipad([r+1 r+2],[c+1 c+2])); im_closing(r,c) = min(m(:)'); end end figure, imshow(uint8(im_closing)); title(['closing ' filename]); end